Compass

Using the magnetometer on the MICA platform

Author:	Peter Volgyesi, ISIS / Vanderbilt University

Date:	July 28, 2026

# How the magnetometer works ?

The MICA sensor board has a magnetometer and some accessory ICs (amplifiers and digital potentiometers) mounted on it. The magnetometer is a HMC1002 chip maid by Honeywell with two axes (X, Y), resolution of ~30 μGauss (Earth’s field is 0.5 μGauss)  and range of -2 to 2 Gauss. The resistance of the axes depends on the direction and strength of the magnetic field around them.

Between the two outputs of each axis there is a small voltage that represents the measured strength of the magnetic field in the given direction. (The voltage is “produced” by a Wheatstone bridge). A typical (healthy) device produces 10mV/Gauss difference. This small voltage is amplified as it is described in the 1.Figure.

10 bits

0 – 3 V

Va1 = 0.92V – 2.08V

290mV/Gauss

Vm1 = 1.5V ± 20mV

10mV/Gauss

Vm2 = 1.5V ± 20mV

Vref = 1.36V – 1.94V

256 steps, 1 step = 5.8mV

Digital potentiometer

A/D

HMC 1002

Magnetometer

AMP

X 29

AMP

X 78

Low Pass Flt

500 Hz

VCC

Figure 1

Values on the figure are based on calculations only !

The amplification is quite large, especially after the second op. amp. Therefore it is very important to set the output of digital potentiometer very close the Va1 voltage. Otherwise, the A/D converter won’t be able to measure the voltage on its output.

Now, to measure the earth’s magnetic filed we will do the following trick:

- We want to set the bias of the whole system to have a measured voltage of ~1.5 V  on the ADC. So, we would like to see a digital value around 512 (BIAS_CENTER) coming from the ADC.

- The achieve our first goal, increase the value of the potentiometer while the ADC value is less then BIAS_CENTER. Every step on the potentiometer will increase or decrease (if we are stepping down) the ADC value by about 32 (BIAS_SCALE).

- Now, the final measured value: ADC_VAL + BIAS_SCALE * BIAS_STEP, where ADC_VAL is the current value measured by the ADC and BIAS_STEP is the digital value of the potentiometer.

Using the previously described simple iteration, we will get a digital value between 0-9000. However, in practice typical values will be between 1500-4500. Unfortunately, every device and every axis will have its own distinct offset and range. (And the sensitivity is degrading by time, also)

So, to build a compass we need a second algorithm, which calibrates itself by measuring the min and max values we have seen on the axes:

- Get the axis readings (magx, magy).

- If magx>MAXX⇒MAXX=magx, magx<MINX⇒MINX=magx

- Execute step 2. for magy

- Calculate x=magx−MAXX+MINX2MAXX−MINX

- Execute step 2. for y

- y>0⇒α=π2−arctg(xy)y<0⇒α=3π2−arctg(xy)

Now, αis our direction (in radians).

# Software Components

The first algorithm is implemented in TinyOS. The module called SmartMagC gives you pairs of values produced by that algorithm. You can set the BIAS_CENTER and BIAS_SCALE (empirical values) on its interface (calibrate(…)).

The second algorithm is implemented in Java (in the GUI subdirectory). You can set the BIAS_CENTER and BIAS_SCALE values from the GUI also.

To use the compass, follow this procedure:

- Compile and install the Compass TinyOS application on a MOTE with  sensorboard. (Make sure, that the magnetometer is installed on the board)

- Compile and install the GenericBase TinyOS application on another MOTE. The first MOTE will send its measurements through radio.

- Compile the java application (Compass.java)

- Turn on the first MOTE and wait 5-10 seconds (it’s going to calibrate itself, and in this period all the values sent by the MOTE are faulty)

- Start the java application, and turn the MOTE around slowly. A whole (360 degree) turn must calibrate the second algorithm.

- Play with the BIAS values, if you wish and have fun. (When you set a new value, the compass MOTE will toggle its yellow LED when it receives these values)

- Enhance the algorithms, and my implementation